Ecology of Mined Watersheds

J. Todd Petty, M. P. Strager, and Paul F. Ziemkiewicz

Over-riding objective:

Develop science-based, spatially-explicit data management and analysis tools that can be used to support decision making (permitting, mitigation, restoration) in mined Appalachian watersheds.

Science-based: field validated empirical models that can predict instream response to past, current, and future land use activities.

- Defendable Thresholds

- Measures of uncertainty

- Interactive effects among stressors

Spatially-explicit: GIS based, networked, multi-scaled watershed models

- Landscape and stream data management

- Statistical models and analysis based on results from watershed research

- Scalable from segment scale to 12- to 10- to 8- digit HUCs

Decision-Support:  Responsive, visual, interactive modeling system

- Maps of current condition landscape (terrestrial and aquatic)

- Identification of dominant stressors, protection priorities, and restoration priorities

- Projections of expected future conditions under alternative management scenarios

- Science-based Alternative Futures Analysis

- The Science

  - Aquatic habitat functions on reclaimed surface mines

  - Monongahela River Studies

    - Linking landscape attributes to instream attributes

    - Quantifying watershed scale consequences of mining on fish assemblages

  - Pigeon Creek Studies

    - Extension of northern coalfield technology to MTR/VF region

    - Identify mining threshold

    - Quantify interactions between mining and residential development

- The GIS modeling system

  - Data management

  - Stream network analysis

  - Scalability

  - Incorporating predictive models (from Petty studies)

  - Visualizing current conditions

  - Projecting future conditions

  - Decision Support features

- Decision Support Applications

  - Maximizing cost-effectiveness of AMD remediation at a watershed scale

  - Predicting Future Conditions in Pigeon Creek Under Alternative Management Scenarios

- Moving Forward

  - Interactive effects of mining intensity, coal geology, mine geography, and residential development in watersheds of the MTR/VF region.

  - Extension of the GIS modeling and decision support system to the MTR/VF region.

  - Alternative Futures Analysis

  - Development of watershed specific management plans for the MTR/VF region.

Summary of “Threshold” Studies

Monongahela River Basin studies

- There is a strong correlation between measures of mining intensity and instream habitat, water quality, and ecological condition metrics in the northern WV coalfields.

- Mining interacts with coal geology and mine geography to influence instream response.

- We identified % mining thresholds that produce localized impairment in streams draining catchments dominated by Freeport coal.

  - 5% mining – a shift from high quality conditions to fair conditions (loss of coldwater fisheries)

  - 20% mining – a shift from unimpaired to impaired conditions (loss of warmwater fisheries)

- Fish assemblages “deflate” at the 10- and 12- digit HUC scales as localized impacts accumulate in a watershed

Deflation is defined as measurable losses of biological condition within stream networks that cannot be explained by local stressors.  In other words, deflation occurs when neighborhood conditions deteriorate and produce reduced biological integrity in streams where local conditions are otherwise good.

- This finding suggests a need to identify a 10- or 12- digit HUC watershed scale deflation threshold of mining that may be different (probably lower) than the thresholds of localized impairment listed above.  Our research suggests that this threshold varies from species to species and probably ranges between 5 and 15% mining for warm-water fisheries.

- Taken together, this means:

  - A mining threshold of 20% is needed to avoid localized impairment for all streams in a watershed.

  - At the outflow of all 10-digt HUCs, a mining threshold of 5-15% is needed to avoid fish meta-community deflation.

Pigeon Creek Studies

- Mining in Pigeon Creek interacts with residential development to influence instream response.

- We identified % mining and development thresholds that produce localized impairment in Pigeon Creek stream segments.

  - 5% mining – a shift from high quality conditions to fair conditions

  - 25% mining – a shift from unimpaired to impaired conditions

  - 5 parcels / km2 – a shift from high quality conditions to fair conditions

  - 15 parcels / km2 – a shift from unimpaired to impaired conditions

- The absolute key finding from this research is that mining and development have an additive effect on instream response.  The thresholds listed above apply only to streams that do not have both stressors present.  When both stressors are present, the % mining threshold declines from 25% to 0% as parcel density increases from zero parcels to 15 parcels / km2.

Remaining Needs

- Quantify mining thresholds across all coal geologies and across a range of residential development conditions throughout the MTR/VF mining region.

- Determine if fish meta-community deflation occurs across a wide range of mined watersheds.

- Alternative Futures Analyses to assess potential for combined management of mining related impacts with restoration of development related impacts in Appalachian watersheds.

Mining Related Publications from WVU Researchers

Peer-Reviewed Manuscripts and Reports Published, In Press, or In Review

Petty, J. T., Fulton, J. B., Merovich, G. T, Jr., Strager, M. P., Stiles, J., and Ziemkiewicz, P. F.  IN REVIEW. Landscape indicators and thresholds of ecological impairment in an intensively mined Appalachian watershed.  Journal of the North American Benthological Society.

Merovich, Jr., G. T., and J. T. Petty. IN REVIEW. Correspondence between stream benthic macroinvertebrates and a discrete disturbance gradient: consequences for diagnosing stressors.  Journal of the North American Benthological Society.

Merriam, E., Merovich, G. T, Jr., and J. T. Petty  IN REVIEW. Additive effects of mining and residential development on stream physical , chemical, and biological conditions in a central Appalachian watershed.  Journal of the North American Benthological Society.

Hense, Z., R. W. Martin, and J. T. Petty.  IN PRESS.  Electrofishing capture efficiencies for common stream fishes in a central Appalachian watershed.  North American Journal of Fisheries Management. 00:000-000.

Strager, M. P., J. J. Fletcher, J. M. Strager, C. B. Yuill, R. N. Eli, J. T. Petty, and S. J. Lamont.  IN PRESS.  Watershed analysis in GIS: the Watershed Characterization and Modeling System Software application.  Journal of Environmental Management 00:000-000.

Martin, R. W., and J. T. Petty.  2009.  Local temperature and thermal topology interact to influence the distribution of brook trout and smallmouth bass in a central Appalachian watershed.  Journal of Freshwater Ecology 23:497-508.

Strager, M. P., J. T. Petty, J. M. Strager, and J. B. Fulton.  2009.  A spatially explicit framework for quantifying downstream hydrologic conditions.  Journal of Environmental Management 90:1854-1863.

Petty, J. T., Brady Gutta, Richard Herd, Jennifer Fulton, James Stiles, Michael Strager, Julie Svetlick, and Paul Ziemkiewicz. 2008. Identifying cost-effective restoration strategies in mining impacted West Virginia watersheds.  Proceedings of the American Society of Mining and Reclamation 25:837-855.

Strager, M.P.,  Vishesh Maskey, J. Todd Petty, Brady Gutta, Jenifer Fulton, Richard Herd, James Stiles, Julie Svetlik, and Paul Ziemkiewicz. 2008.  A hydrologically networked watershed model for evaluating AMD treatment scenarios.  Proceedings of the American Society of Mining and Reclamation 25:1217-1231.

Freund, J. G. and J. T. Petty.  2007.  Response of fish and macroinvertebrate bioindices to specific stressor levels in a mined Appalachian watershed.  Environmental Management 39:707-720.

McClurg, S., J. T. Petty, P. M. Mazik, and J. Clayton. 2007.  Stream ecosystem response to limestone remediation programs watersheds of the Allegheny Plateau ecoregion.  Ecological Applications 17: 1087-1104.

Merovich, Jr., G. T., and J. T. Petty. 2007. Interactive effects of multiple stressors and restoration priorities in a mined Appalachian watershed. Hydrobiologia 575:13-31.

Merovich, Jr., G. T., J. M. Stiles, J. T. Petty, J. Fulton, and P. F. Ziemkiewicz. 2007. Water chemistry based classification of streams and implications for restoring mined Appalachian watersheds. Environmental Toxicology and Chemistry 26:49-57.

West Virginia Water Research Institute. 2007. Abram Creek Watershed Restoration Plan. Report prepared for the West Virginia Department of Environmental Protection Abandoned Mine Land Program.

West Virginia Water Research Institute. 2007. Three Fork Creek Watershed Restoration Plan. Report prepared for the West Virginia Department of Environmental Protection Abandoned Mine Land Program.

West Virginia Water Research Institute. 2007. Paint Creek Watershed Restoration Plan. Report prepared for the West Virginia Department of Environmental Protection Abandoned Mine Land Program.

Petty, J. T., and D. Thorne. 2005.  An ecological currency to identify restoration priorities in an acidic Appalachian watershed.  Restoration Ecology 13(2): 348-357.

Petty, J. T., and J. Barker.  2004.  Water quality variability, trace metals, and implications for restoring a mined Appalachian watershed. Proceedings of the American Society of Mining and Reclamation 21:1484-1504.

Peer-Reviewed Manuscripts in Preparation

Hense, Z., and J. T. Petty.  IN PREP.  Juvenile and adult fish distributions along a central Appalachian river continuum.  Freshwater Biology.

Martin, R. W., and J. T. Petty.  IN PREP. Evidence of regional – impairment to fish assemblages in an intensively mined Appalachian watershed.  Ecological Applications.

Martin, R. W., and J. T. Petty.  IN PREP.  A “house-neighborhood” view of stream fish assemblages and the consequences of watershed scale impairment to aquatic communities.  Canadian Journal of Fisheries and Aquatic Sciences.

Petty, J. T., G. T. Merovich, Jr., J. B. Fulton, M. P. Strager, and P. F. Ziemkiewicz.  IN PREP.  Setting protection and restoration priorities at multiple scales in intensively mined Appalachian watersheds.  Ecological Applications.

Graduate Student Theses and Dissertations

Martin, R. W.  IN PREP.  Local and regional influences on fish community structure in acid-impaired watersheds of north central West Virginia.  PhD Dissertation.  West Virginia University, Morgantown, WV.

Gingerich, G. 2009.  Quantifying changes in ecological function of headwater catchments following large-scale surface mining in southern West Virginia. MS thesis. West Virginia University, Morgantown, WV.

Merriam, E. 2009.  Mining and residential development interact to produced highly impaired stream conditions in an intensively mined Appalachian watershed.  MS thesis. West Virginia University, Morgantown, WV.

Minter, M. 2009.  Organic matter processing and opportunities for mitigation in an intensively mined West Virginia watershed.  MS thesis. West Virginia University, Morgantown, WV.

Merovich, Jr., G. T. 2007.  Stream water quality and benthic macroinvertebrate ecology in a coal-mining, acid sensitive region.  PhD Dissertation.  West Virginia University, Morgantown, WV.

Hense, Z.  2007.  Stream fish populations in a watershed scale context for fish community dynamics in central Appalachian watersheds.  Master’s Thesis.  West Virginia University, Morgantown, WV.

Freund, J. G.  2004.  Local and regional impairment of fish communities in a mined Appalachian watershed.  PhD Dissertation.  West Virginia University, Morgantown, WV.

Martin, R. W.  2004.  Stream thermal geography and the distribution of brook trout and smallmouth bass in the Cheat River watershed, WV.  Master’s Thesis.  West Virginia University, Morgantown, WV.

McClurg, S. E.  2004.  Stream ecosystem response to mitigative limestone treatment in acid impaired, central Appalachian streams.  Master’s Thesis.  West Virginia University, Morgantown, WV.